Aum Crematorium

Technical note on emission limits, pollutant capture and collected residues

| Purpose | Regulatory and technical overview |
| --- | --- |
| Scope | FT4 cremator with dry flue-gas treatment / fabric filtration |
| Status | Guidance |

# 1. Purpose and regulatory context

This technical note summaries the requirements most relevant to an FT4 cremator equipped with flue-gas treatment. It provides a chart-based view of the PG5/2 (25) emission limits, explains the function of the FT4 filtration system, and identifies the particulate and adsorbed materials expected within the collected filter residue.

PG5/2 (25) is the current UK statutory process guidance used by local authority regulators when assessing applications and writing permits for crematoria. The guidance was published on 4 December 2025 and updated on 7 July 2026. Existing cremators must comply with the emission limit values for new cremators from 4 December 2029.

# 2. PG5/2 (25) emission limits: chart view

Table 1 presents the limits for cremators other than unabated cremators. Values are stated at 273.1 K, 101.3 kPa, 11% oxygen by volume, dry gas. Except for PCDD/F, compliance is assessed using the average of three consecutive 60-minute measurements.

| Substance | Existing cremator | New cremator | Minimum monitoring | FT4 control route |
| --- | --- | --- | --- | --- |
| Particulate matter | 10 mg/Nm³ | 5 mg/Nm³ | Annual periodic monitoring | Controlled by fabric filtration |
| Hydrogen chloride (HCl) | 30 mg/Nm³ | 20 mg/Nm³ | Annual periodic monitoring | Acid-gas reaction and filtration |
| Total organic carbon (TOC) | 20 mg/Nm³ | 10 mg/Nm³ | Annual periodic monitoring | Primarily combustion control; activated carbon supports capture of organic species |
| Oxides of nitrogen (NOx) | 200 mg/Nm³ | 200 mg/Nm³ | Annual from 1 month after publication | Not removed by standard filter; separate SNCR may be required |
| Mercury | 50 µg/Nm³ | 30 µg/Nm³ | Annual periodic monitoring | Adsorption onto activated carbon |
| Dioxins and furans (PCDD/F) | 0.1 ng/Nm³ | 0.1 ng/Nm³ | Every 3 years; annual in specified cases | Adsorption onto activated carbon and retained by filter |
| Ammonia (NH₃) | No ELV | No ELV | Annual only where SNCR is installed | Measures ammonia slip from NOx abatement |

Table 1. PG5/2 (25) Table 6 requirements relevant to an FT4 installation.

Operational parameters: Carbon monoxide is an operating performance target of less than 100 mg/Nm³, not a Table 6 emission limit. Oxygen, secondary chamber temperature, and residence time are operational controls. For cremators with flue-gas treatment, the normal minimum secondary-chamber temperature is 800°C, rising to 850°C if the treatment equipment fails, unless different permit conditions are authorised. Minimum residence time is 2 seconds.

# 3. FT4 flue-gas treatment and filtration system

The FT4 arrangement considered in this note uses a dry flue-gas treatment process. The combustion gases are cooled, alkaline reagent and activated carbon are introduced, and the resulting solids are retained by a fabric filter. The fabric filter therefore performs two related functions: direct removal of entrained fly ash and retention of spent reagent carrying reacted or adsorbed pollutants.

| Combustion | → | Cooling | → | Reagent dosing | → | Fabric filter |
| --- | --- | --- | --- | --- | --- | --- |

## 3.1 Pollutant-control functions

| Pollutant / parameter | FT4 treatment mechanism | Outcome |
| --- | --- | --- |
| Particulate matter / cremator fly ash | Physical capture by the fabric filter | Collected as solid filter residue. |
| Hydrogen chloride and other acid gases | Reaction with alkaline reagent, followed by filtration | Forms solid sodium salts where sodium-based reagent is used. |
| Mercury | Adsorption onto activated carbon | Mercury-bearing carbon is retained on the filter cake. |
| Dioxins and furans | Adsorption onto activated carbon | Adsorbed organic micropollutants remain associated with spent carbon. |
| Hydrocarbons / chlorinated hydrocarbons | Adsorption onto activated carbon | Organic compounds may be stabilised by adsorption onto the carbon surface and removed with the filter residue. |
| NOx | No material removal by the standard dry filter | Requires combustion optimisation and, where installed, separate SNCR. |
| Carbon monoxide | No material removal by the filter | Controlled through good combustion and monitored as an operating target. |

# 4. Materials captured within the FT4 filter residue

The collected residue is a mixed solid stream. Its composition depends on the reagent formulation, dosing rate, coffin, and cremation inputs, combustion conditions, and filter performance. The following chart incorporates the specified constituents and explains their expected origin or retention mechanism.

| Material | Origin / capture mechanism | Status in residue |
| --- | --- | --- |
| Sodium chloride (NaCl) | Neutralisation product associated with capture of hydrogen chloride by sodium-based alkaline reagent. | Solid salt retained in filter cake. |
| Sodium sulphate (Na₂SO₄) | Oxidised sulphur-containing neutralisation product. | Solid salt retained in filter cake. |
| Sodium nitrate (NaNO₃) | Potential sodium salt associated with nitrogen-containing acid species. | May be present in the mixed residue; confirm by analysis. |
| Sodium sulphite (Na₂SO₃) | Partially oxidised sulphur-containing neutralisation product. | Solid salt that may oxidise further to sulphate. |
| Cremator fly ash | Fine mineral and combustion-derived particulate entrained in the flue gas. | Physically captured by the fabric filter. |
| Activated carbon | Injected adsorbent retained within the filter cake. | Carries absorb pollutants and become part of the spent residue. |
| Hydrocarbons | Organic compounds absorbed onto activated carbon. | Stabilised on the carbon surface and removed with spent carbon. |
| Mercury | Adsorbed onto activated carbon; a smaller fraction may be associated with particulate. | Retained in the carbon/filter cake residue. |
| Dioxins and furans (PCDD/F) | Micropollutants adsorbed onto activated carbon. | Retained with spent activated carbon. |
| Chlorinated hydrocarbons | Organic chlorine compounds adsorbed onto activated carbon. | Stabilised on the activated-carbon surface and removed with the residue. |

# 5. Monitoring and assurance

| Parameter | Frequency | Purpose / standard |
| --- | --- | --- |
| Dust arrestment plant monitor | Continuous | Required for crematoria with flue-gas treatment to detect failures likely to cause a particulate ELV exceedance. |
| Particulate matter | Annual | Periodic test to EN 13284-1. |
| Mercury | Annual | Periodic test to EN 13211. |
| Hydrogen chloride | Annual | Periodic test to EN 1911. |
| Total organic carbon | Annual | Periodic test to EN 12619. |
| NOx | Annual | Periodic test to EN 14792, from one month after publication. |
| PCDD/F | Every 3 years normally | Annual for unabated cremators and where alternative operating conditions are authorised; first new cremator measurement within 12 months. |
| CO and oxygen | Continuous operational monitoring | Used to demonstrate combustion performance and operational control. |

# 6. Conclusion

For an FT4 cremator fitted with dry flue-gas treatment, the fabric filter is the principal particulate-control stage and captures spent alkaline reagent and activated carbon. The collected residue may therefore contain sodium chloride, sodium sulphate, sodium nitrate, sodium sulphite, cremator fly ash and activated carbon carrying adsorbed hydrocarbons, mercury, dioxins, furans and chlorinated hydrocarbons. These materials are removed from the flue-gas stream as a combined solid residue.